The City of Lafayette    Water Works Plant

Annual Report

2019

Ron Hurst: Assistant Superintendent

Lafayette Water Works

Mission Statement

The Lafayette Water Works is a municipally owned and operated water Utility. It is our purpose under the direction of the Mayor, Common Council, Board of Public Works and Safety and Superintendent, to provide its customers with potable water, with adequate pressure, quality and quantity and to work with the Fire Department to maintain the fire hydrants and distribution system that provides fire protection to its customers.

To accomplish this purpose we will pursue professional ethics that promote public health and safety, consumer confidence, customer service, responsible operations, preventative maintenance, accurate laboratory testing, reliable reporting and compliance with EPA and the Indiana Department of Environmental Management Regulations, AWWA Standards and other safety programs, showing fiscal responsibility to maintain the best user rate possible for our customers.

Joe Davenport:  Backflow/Cross Connection Inspector

The Backflow/Cross Connection Inspector is responsible for tracking the annual backflow test results for all backflow devices in the City of Lafayette’s potable water system, performing annual site surveys to identify any new hazards at a facility, assist in identifying the appropriate backflow device(s) required in new construction, tracking the annual gauge calibration certificates for all backflow testers who submit results and helping with continuing education of local backflow device testers.

- Performed 17 Site Surveys

- Performed 335 Inspections

- Delivered 12 Notices To Test backflow assemblies

- Delivered 1 Notices To Install backflow assemblies

- Helped Water Facility Inspector as needed

- Helped to log and file incoming maps

- Performed 1 Hydrostatic Test

- Performed 6 Bacteria Tests

- Filled in for our Utility Locators as needed

- Made corrections to ARC/GIS and MyGIS manager online mapping

- Continuing to verify fire line sizes for our Utility Billing Dept.

- Continuing to update and maintain our online Database of Backflow assembly test results through BSIonline and in our Laserfiche system

- Conducted a Site Visit and Tour, at the Water Works Canal Rd. Pumping Station.

- Gave a condensed Water Works Presentation to Faith School at City Hall.

- Gave a Water Works Presentation to the Purdue Retirees at MCL Cafeteria

- Visited BSI’s main office to discuss issues with the database and logging test results

- Oversaw Irrigation startup, winterization and maintenance

- Attended ADA Meetings

- Assisted with Fire Line Inspections at City Buildings

Steve Moore:  Chief of Distribution

The Construction Department was able to assist other City Departments on many occasions in 2019. We help other departments with snow removal by plowing two routes on the North end.  We also work closely with the Street Department during street paving to assure that we replace any broken valves or hydrants “before” the street is repaved. The Water, Street, Fire, Parks and Waste Water Departments have a long history of working together.  In return, we have received help from them on any occasion that we have asked.

Distribution System Accomplishments for 2019

- Fire hydrants replaced	13

- Fire hydrants repaired	           33

- Leaks repaired		           42

- Broken mains repaired	 56

- Retire old lead services          28

- New services		           180

- Side-taps                              36

- 1” taps                                 12

- Updated Meter pits	            17

- Salt pumps		            0

- Valves replaced		  2

- Valves Repaired                     1

- Replaced 5,255 linear ft. of 6” cast iron main with new 8” ductile iron main on Longlois and Vinton St. Replaced 131 service connections, 12 line valves and 8 fire hydrants.

Utility Locator:

Utility Locators perform an invaluable service for the Water Works and the City of Lafayette.  They electronically locate the City’s water mains and mark them on the ground with blue paint/flags.  The Utility Locators are also responsible for marking the City owned fiber conduits along Greenbush Street and Brady Lane with orange paint/flags.  Without these locations there could easily be a large service outage to business and citizens of Greater Lafayette and surrounding areas. Locators also respond to emergency locates that are called in after hours and on weekends and holidays.

In house projects include; maintaining the map log conversion to the grid system, log new maps into the map logs and grid system, complete ARC GIS corrections, updating the fire system, placing fire hydrant marker sticks on fire hydrants that are not easily seen,

Utility Locators 2019 Accomplishments:

  - Reviewed and logged into the system  new maps

  - Drew new maps for the Distribution System.

  - Number of utility locates completed 13,524 responded to 588 emergency locate requests and assisted contractors working in and around the city with locate information.

  - Assisting Intra Water Works/Other City Departments.

  - Continuation of the ARC GIS corrections	.

Hydrant Flushing Program:

Each year, 6 workers flush nearly 3300 fire hydrants in the spring and fall flushing programs. Flushing mains help to determine what is happening in the system and shows any buildup of iron or manganese in specific areas. This is also an opportunity to operate the hydrants to assess the need for maintenance or repair.  A census of the hydrants is also made at this time, which allows updating of information used by other City Departments.

# Water Works Laboratory

Wellhead Protection Program

As part of the Well Head Protection program, wells at the Canal and Glick well fields and the adjacent old City Golf Course are sampled and tested for volatile organic compounds. Nine Canal & five Glick production wells are tested once a year. Forty-one monitoring wells are tested monthly and bi-annually for volatile organic compounds at the old City Golf Course. Samples are sent to independent certified labs for analysis.

As the Water Works Lab Technician, we keep updated files of all WHP test results, updating and comparing results monthly.  We also complete monthly IDEM Reports.

Lead and Copper Project

In 2019, 30 homes were tested for lead and copper. As in the past we organize and supervise this program, communicating with a certified chemistry lab. We also supply residents with preserved sample bottles and collect paperwork. Results are reported to IDEM and test results are mailed to all residents.

Results: Ninety percent of samples were at or below:

| 2019 | Lafayette | EPA’s Maximum Allowed |
| --- | --- | --- |
| COPPER | .672 mg/L | 1.3 ppm |
| LEAD | 1.1 ug/L | 15 ppb |

Lafayette adds a specially blended polyphosphate designed to minimize corrosion scale, and red water conditions (Iron). Since we continue to be in compliance with State and EPA lead and copper regulations, we have been granted “reduced monitoring” status for the upcoming years, testing once every three years.

Consumer Confidence Report (CCR)

2019 was the 22nd year for the annual water quality report for customers or “Consumer Confidence Report.  The finished report contains water quality information and testing results, including charts explaining the presence of any man-made or natural chemicals, minerals, etc. Information is also supplied to the Town of Dayton.  Even though they purchase water from us, they still need to create their own CCR Report. The report was approved by IDEM, mailed out with water bills and posted on the Internet.

Bacteria Testing in the Distribution System

Public water systems must collect total Coliform samples at sites that are representative of water throughout the distribution system.  This is done according to a written sitting plan approved by the commissioner. The monitoring frequency for total Coliforms for community water systems is based on the population served by the system.  This requires Lafayette Water Works to collect 70 bacteria samples a month, derived from population base (59,001-99,000). We monitor within the outer boundary of the system monthly. These samples are sent to a State Certified Lab, and tested for Total Coliforms. The test results are faxed to IDEM within forty-eight hours, and are kept on file.

Other Duties

- Oversee daily operation of lab and equipment

- Keep inventory and order supplies

- Prepare reagents

- Keep accurate, up-to-date records

- Keep up on current and proposed regulations

- Create new report forms, charts, graphs, etc.

- Quality control tests

- Responsible for various reports:

  - Monthly Report Operations(MRO)

  - Daily Lab Report

  - Lead and Copper Report(Pb-Cu)

  - Consumer Confidence Report(CCR)

  - THM’s & HAA5 (Trihalomethanes-Haloacetic Acids)

### 2019 Water Sampling and Analysis

TEST				SITES				FREQUENCY

| Lead and Copper | Residents’ homes | 30/ Every three yrs. |
| --- | --- | --- |
| 42 Volatile Organics | Individual Canal and Glick Wells | Monthly—Total of about 120 in 2019 |
| 42 Volatile Organics | Canal and Glick Entry Points | 2nd Qtr. Only. ( Waiver) |
| Bacteria Samples | Distribution System | 70/ month—840 in 2019 |
| Alkalinity, pH, Phosphate, Temperature | Distribution System | 10 sites 4X year |
| 13 Inorganics | Canal and Glick Entry Points | Waiver/ Every three yrs. |
| Synthetic Organic | Canal and Glick Entry Points | Reduced monitoring |
| THM’s /HAA5 | Distribution System | 4 / Sites per Qtr. |
| Combined Chlorine, Phosphate, Fluoride, pH | Canal and Glick Entry Points | 4X Daily (by System Operators) |

Pat Schultz:  Maintenance Foreman

Staff

The Maintenance Department consists of one maintenance foreman and three maintenance personnel.

The main duties of this Department are to effectively monitor and perform maintenance on all the pumping equipment, recognizing small problems before they escalate to larger ones. We perform daily, bi-weekly, monthly, bi-monthly, quarterly, bi-annual, and annual testing on our pumping equipment, generators and the chemical feed systems.  We tabulate the combined data to determine which wells and pumps are the best candidates for the Annual Well Rehabilitation Program. The data collecting, record keeping, presentation, and well maintenance forms have been acclaimed as the best in the State.

A preventive maintenance program is performed on all pumping equipment and chemical feed equipment, such as oil changes, greasing, calibration, and signal checking. The Maintenance staff also has the duties of Water Tower calibration, rinsing, cleaning and proper illumination and security testing.

Our staff also assists with security maintenance, installation of security lighting, repairing of motion detectors, testing of devices such as battery efficiency, and lighting devices.  We are on call twenty-four hours a day, seven days a week. We have responded to calls such as SCADA malfunctions, pump motor problems, automatic valves not reacting properly, etc.

We have worked with the Distribution Department on repairing broken mains, unplugging water lines and repairing service line leaks. We have also assisted in helping the Customer Service Department in reading meters during severe weather conditions along with installing new water meters and snow plowing.

The staff also assists in the general up-keep of the Water Departments facilities, electric, plumbing, HVAC, and the buildings appearance.

The Maintenance Department goals are to provide a safe and continuous flow of quality water to our customers. Our thorough Well and Pump Testing, as well as our excellent preventive maintenance programs allow the Water Works Department to deliver the highest in quality and most cost efficient product to our customers.

This year we converted fluorescent lights to LED at some locations at a substantial cost savings on lights and energy usage.

Inspections and Reports:  Daily, Weekly, Bi-Weekly, Monthly, Quarterly, Annually

- Wells and Pumps, Flows, Meters, Electrical, Packing, Construction Preventative,       Alignments, Towers

- Water Treatment Systems, Chlorine equipment,

- Electrical ~ Emergency Generators, Lighting & Batteries

- Tools ~ Inventory for Vehicles and Storage

- Calibration and testing Chlorine and Hydrogen sensors

Wells, Pumps & Gen-Sets

- Changed oil, oil filters and fuel filters in all three Gen-Sets

- Cleaned all drain-back valves, tested

- Assisted with annual hoist inspections

- Changed motor oil at all wells

- Replace motor on well #4 at canal

- Packing adjusted at all wells

- Removed and replaced one leg of 3 phase wire on C8.

- Replaced AC/DC converter at glick#5 well.

- Sanded rust off 480V 3phase starter contactors in all electric 480v panels.

- Install and remove insulation and heat cables from all wells.

- Diagnose and replace as needed batteries on gen sets.

Water Treatment Process

- Cleaned flow meters at all pumping stations and replaced whole system at Glick Station.

- Repaired and maintained phosphate, chlorine, Re-Ox, fluoride & ammonium sulfate systems

- Maintain and repair as needed annually on Hypo-Generator.

- Installed pump motor for pumping ammonia into system.

- Clean walls, ceilings, floors, tanks and modules after chlorine leaks.

- Drained 5000 gallon tank of chlorine cleaned and installed new shut offs and drain seals.

Water Towers/Reservoir

- Maintain and repair Ethernet cables and connectors for communication for scada.

- Monthly tower inspections

- Converted fluorescent lights into LED.

- Replaced schedule 80 drain pipe at Haggarty Tower.

- Replaced electric motors in heaters for steel water towers.

- Maintain Fences at towers replacing fence barbs, strap anchors and gate hinges

and locks.

- Drained and flushed towers as needed.

Electrical

- Maintain and replace solenoids, micro switches, in vaults and pumps @ pumping stations.

- Replaced sump pumps in vaults at wells, vaults in pumping stations

- Replaced battery charger in Genie Lift.

- Repaired circuit board on prominent pump for fluoride at Chemical Feed building.

- Installed electric and lighting from Chemical Feed building to 16X30 shed.

Building Maintenance

- Repaired various security lighting

- Assisted with annual hoist inspections at all facilities

- Converted fluorescent lights to LED at Columbian Park pump station.

- Installed new ballasts in various lighting fixtures

- Replaced filters and cleaned AC units at all facilities

- Changed oil in all air compressors

- Keeping sidewalks and entrances safe at water works locations during the winter

- Repaired automatic louvers when exhaust fan is on to exhaust fume to the outside of chemical feed building.

- Bricked up hole in wall where A/C was removed in electric room in chemical feed building.

- Installed digital door handle lock on shed service door behind chemical feed building.

- Installed new LED outdoor lights on shipping and receiving building at canal.

- Replaced transformer above front door at chemical feed building at canal.

- Replaced boiler heat circulator coupling on pump for boiler heat at Columbian Park.

- Ran cat 6 Ethernet cable and installed security camera outside at Columbian Park pump building.

- Perform maintenance and repairs on security systems at all water works locations.

- Provide repairs for safety at locations as an employee tripped over raised concrete at joints in walkway and we ground joints and sealed them flush to prevent an accident from happening again.

- Installed decking around parts of new shed at Chemical Feed Building at Canal.

SCADA

- Provide maintenance on SCADA system and repair during the year.

Irrigation Maintenance

- Annual maintenance and repairs on irrigations as needed.

Andrew Moore:  Operations and Lab Foreman

STAFF

The Operations Department consists of one Operations & Lab Foreman, four System Operators, and one Lab Technician. All staff members, except the recently hired System Operator, retain WT2 or higher Water Treatment licenses. The Foreman, one System Operator, and Lab Technician all retain DSL Large Distribution System licenses.

Service

We provide service twenty-four hours a day, seven days a week, with assistance of an ‘on-call’ person. We received at least 2160 service calls in the year 2019, which averages out to 180 calls a month. These calls occurred during nights, weekends, and holidays, and consisted of turning on water service, changing out water meter chambers and screens, thawing frozen meters, and much more. There are emergency service requests that we respond to consisting of everything from broken mains, water pipe breaks in homes and businesses, and fire hydrants that have been broken off by vehicles, etc. We also assist the Customer Service Department, Distribution Department, and Maintenance Department here at the Water Works, plus assist the Police, Utility Billing, Facilities Maintenance, Parks, Water Pollution Control, and the Street Departments’ when they are in need of assistance.

Quality

We perform, at a minimum, two rounds of inspections and tests per eight hour shift to ensure water quality and to ensure proper water treatment process operation at Canal Road Well Field, Glick Well Field, and Columbian Park Booster Pump Station. Cl2 readings are taken at two remote locations within the city and all 14 wells and 6 booster pumps are inspected for proper pumping operation. Building Maintenance/Upkeep is a large undertaking and we make sure janitorial responsibilities are met, painting is kept up, and everything is in good repair.

We monitor, with the assistance of our Supervisory Control And Data Acquisition (SCADA) system, the water levels in our (1) five million gallon enclosed reservoir located at Columbian Park, our (1) two million gallon elevated water tower located at Haggerty Lane, and our (3) one million gallon elevated water towers located at Union Street, Fairgrounds, and Plaza South. The system will alarm when a problem occurs with high/low water levels, well and pump failures, electrical power failures, and security. If the operator is away from the office the SCADA system will send notifications to a smart-phone carried by the operator to let them know about problems as they occur.

Security

On a daily basis we provide a level of security at all locations making sure the areas are well lit, locked, and that security cameras are working properly. We ensure security systems at each of our locations are working properly by logging into a smart-phone application and making sure they are armed and ready at the end of each day.

Aim

We in Operations are dedicated to moving the City of Lafayette Water Works Operations Department forward in giving our customers the best quality water possible, by keeping up with Federal and State regulations, streamlining our processes for better efficiency, providing the customers the quantities of water needed by maintaining suitable water levels, assisting with keeping our production of water up, and improving and updating our water system’s security from attack.

Summary

During the second half of 2019 we had a System Operator move to a different department. A new person with no prior operating experience was hired to fill the position. This resulted in a large amount of time being dedicated to training. Also, with the retirement of our Lab Technician in view, a lot of 2019 was spent training a replacement on lab practices, State regulations, record keeping, and familiarity with sampling sites and procedures.

In summary of the larger accomplishments achieved; in 2019 we spent a great deal of time with engineers during the planning phases of a new 3 million gallon reservoir and pumping station at Murdock Park, a new 2 million gallon water tower near Wea Ridge Elementary School, as well as planning for significant electrical upgrades to our existing Columbian Park Booster Pump Station. In the last half of 2019 we also began working with an engineer on redesigning and upgrading our Canal chemical treatment building. These upgrades are centered on meeting newer State standards on chemical containment, but also to make necessary upgrades to capacity needed to meet dosage requirements. These upgrades should be implemented in two phases throughout 2020 and 2021.

On the plant automation side of things, we are still working with SCADA engineers to update the alarming and notification systems for our treatment operations, as well as completing the overall system upgrade began in late 2018. Planning for the SCADA additions of new distribution system sites, such as the Murdock Park Reservoir, will continue throughout 2020.

Along with maintaining our day-to-day operations, we have assisted the Customer Service Department with disconnects, service calls, meter-reading, and the meter change-out program. During winter months, all System Operators and the Operations & Lab Foreman are available to assist with snow removal as well as water main repairs if necessary.

City of Lafayette Water Works

2019 Usage Report. GALLONS PUMPED; TOTAL GALLONS; GALLONS REPUMPED. MONTH; CANAL; GLICK; CANAL & GLICK; PARK. January; 186,990,690; 173,074; 187,163,764; 150,894,949. February; 182,333,602; 0; 182,333,602; 143,606,142. March; 142,803,342; 95,311,352; 238,114,694; 147,115,766. April; 70,020,346; 222,298,161; 292,318,507; 149,719,553. May; 88,992,610; 231,874,433; 320,867,043; 172,099,407. June; 82,968,042; 247,224,543; 330,192,585; 173,152,487. July; 112,990,857; 257,780,905; 370,771,762; 208,714,636. August; 108,744,962; 267,795,793; 376,540,755; 207,695,577. September; 140,326,180; 146,447,441; 286,773,621; 184,038,935. October; 202,259,086; 0; 202,259,086; 181,305,494. November; 172,201,408; 0; 172,201,408; 154,492,107. December; 143,382,223; 74,228,841; 217,611,064; 160,223,837. Yearly Total; 1,634,013,348; 1,543,134,543; 3,177,147,891; 2,033,058,890. Monthly Average; 136,167,779; 128,594,545; 264,762,324; 169,421,574. Monthly Minimum; 70,020,346; 0; 172,201,408; 143,606,142. Monthly Maximum; 202,259,086; 267,795,793; 376,540,755; 208,714,636.

City of Lafayette Water Works. Production Report 2019. Year; Annual Pumpage Canal; Annual Pumpage Glick; Total Annual Pumpage; Daily Average; Maximum Day Pumpage. 2014; 983,135,872; 1,763,795,924; 2,746,931,796; 7,849,298; 10,965,016. 2015; 1,740,452,106; 1,423,177,936; 3,163,630,042; 8,667,479; 14,348,981. 2016; 1,412,890,000; 1,841,200,000; 3,254,090,000; 8,915,315; 16,655,283. 2017; 1,714,575,000; 1,555,369,000; 3,270,217,000; 8,959,498; 15,880,935. 2018; 2,346,018,700; 1,036,479,100; 3,382,497,800; 9,267,117; 13,043,083.

Public Works Inspectors:

The Inspector for the Lafayette Water Works Plant is responsible for hydrostatic, bacteria testing and facility inspections.  These duties are performed according to a set of guidelines to assure that all new water connections are done according to City Standards.

  - Performed 33 Hydrostatic tests

  - Performed 202 Bacteria tests

  - Performed 3 Well Separation inspection

  - Attended 11 Preconstruction Meetings

  - Reviewed new maps

  - Inspected and checked right of ways on 24 fire hydrants

  - Performed 433 site and facility inspections

  - Performed 48 meter and meter pit inspections

  - Performed 21 Hard flush on Fire lines

  - Assisted with utility line locations

  - Assisted Construction and Meter Department crews

  - Filled 40 systems

  - Witnessed chlorination 28

  - Witnessed dechlorination 32

  - Assisted Customer Service as needed

  - Flushed system 145

  - Attended Training Events 3

  - Turn off 24 / Turn on 52

  - Valve located / repaired 9

Steve Moore:  Safety Officer for the City of Lafayette and Chairperson of the Safety Steering Committee

As Safety Officer I am responsible for the enforcement of all City safety policies. As Chairperson for the Safety Steering Committee I preside over a Committee that is comprised of representatives from each City Department. We meet monthly at The Water Works Plant to discuss activity from Departmental Safety Committees; there we explore ways to prevent accidents to ensure the safety of all City Employees.

Mission Statement

It is the mission of the Safety Steering Committee of the City of Lafayette to increase and maintain the commitment of employees in health and safety issues; to increase awareness that supervisors and employees are primarily responsible for the prevention of workplace accidents; to make safety activities an integral part of the City of Lafayette`s procedures and culture; to provide an opportunity for the free discussion of safety problems and solutions; to help reduce the risks of workplace injuries; and to help ensure compliance with federal and state health and safety standards.

Chuck Reynolds:  Chief of Customer Service

The Customer Service team has served our water customers in a multitude of ways in 2019. There were 21,458 customer service orders that were performed. Consisting of frozen meters, need reading, check for leaks, verify meter information, weekly checks to monitor consumption, check for stopped meter, check for damaged equipment, application and finals for new and existing services, repair meter, replace antenna, replace meter, test meter, repair or replace lid or bell assembly, and replace battery or MIU radio read unit. We also disconnected 4,282 services for non-payment. Are Customer Service team also changed out 1,769 Sensus brand meters to Neptune R900-I radio read units. The Customer Service team is made up of nine members who are dedicated to serving the 29,651 metered accounts that we currently have in an efficient and professional way. Some team members are also in a support role for snow plowing and broken main repair. During the warmer months we maintain lawn maintenance on various City of Lafayette properties to ensure an excellent reflection on our community and our customers.